Build Your Own

NeckWatch

Using the Upside Down Kit + Your Laptop

A complete beginner's guide, no experience needed! By the end of this guide you will have NeckWatch, a real-time EMG alert system that beeps and lights up a live dashboard every time your posture goes wrong at your desk.

What you will need from your kit:

| Hardware from kit | What it does in this project |
| --- | --- |
| Muscle BioAmp Shield v0.3 | Amplifies your muscle signal (the brain of the circuit) |
| Arduino Uno R4 + USB-C cable | Reads the signal and talks to your laptop |
| BioAmp Cable (100cm) | Connects electrodes to the shield |
| 3 Jumper cables | Links shield pins to Arduino if needed |
| 3 Gel Electrodes | Stick to your skin to pick up muscle signals |
| Electrode Gel + NuPrep Gel | Makes the electrode connection much cleaner |
| Skin Wipes | Clean your skin before placing electrodes |

# Section 1: How Does This All Work?

Before you touch any wires, let's understand what we're actually building. Don't skip this part, it will make everything else make sense!

## 1.1  What is a Muscle Signal?

Your muscles are powered by electricity. Every time your brain tells a muscle to contract (squeeze), it sends a tiny electrical signal through your nerves to that muscle. Those signals are incredibly small (we're talking millionths of a volt), but they're real and we can measure them.

This type of signal is called an EMG, short for Electromyography (say it: electro-my-OG-raf-ee). Scientists and doctors have been using EMG for decades to study muscles.

Why does bad posture show up in EMG? When you sit up straight, the muscles in the back of your neck and upper shoulders (called the trapezius muscles) are in a nice, relaxed state. When you hunch forward over your laptop (like most of us do), those same muscles have to work extra hard to hold your head up. Your head weighs about 5 kg (11 pounds)! The further it tilts forward, the more your muscles strain. This strain shows up as a bigger, more sustained EMG signal. Our software will detect that and warn you before it causes pain.

## 1.2  What is the Muscle BioAmp Shield?

The tiny electrical signal from your muscles is way too small for an Arduino to read directly. The Muscle BioAmp Shield is a special circuit board that:

- Amplifies the signal (makes it bigger), about 1,000 times bigger!

- Filters out noise, things like power-line interference from nearby plugs

- Cleans it up so the Arduino can read it clearly

It plugs right on top of the Arduino Uno, like a hat. The two boards together become your measurement computer.

## 1.3  What does the Arduino do?

The Arduino Uno R4 is a tiny computer, about the size of a credit card. It has a chip called an ADC (Analog-to-Digital Converter) that turns the smooth electrical wave from your muscles into numbers from 0 to 1,023 (10-bit resolution), sampled every 2 milliseconds (about 476 times a second). Those numbers get sent through the USB cable to your laptop, where Python reads them.

## 1.4  What does the Python script do?

Python is a programming language that runs on your laptop. The posture_monitor.py script will:

- Auto-detect and connect to the Arduino over USB (or use a COM port you set manually)

- Read the six comma-separated values the Arduino sends each sample: the raw signal, three RMS windows (short/long/trend), a posture score, and a bend flag

- Blend those three RMS windows into a single 0 to 100 posture score, weighted toward sustained tension rather than quick twitches like swallowing or talking

- Fire an alert (a two-tone beep) once the score has stayed above threshold for a set hold time, with a cooldown so it doesn't fire again immediately

- Draw a live dashboard: the raw EMG trace on top, the posture score history below, plus a session timer, peak-score readout, and alert counter

What is RMS? RMS stands for Root Mean Square. It sounds scary but the idea is simple: it's a way of measuring the average 'strength' of a signal that goes up and down. Imagine you're measuring how loud music is. The sound wave goes up and down really fast. RMS gives you one number that represents how loud it feels overall. For your muscles, a high RMS means your muscles are working hard, which means bad posture!

# Section 2: Hardware Setup (The Physical Stuff)

OK, time to get hands-on! This section takes about 15 minutes. Go slowly and follow each step exactly.

Safety note: read this first! The signals we're measuring are coming FROM your body. We're not putting any electricity into you. This is totally safe. The kit runs on the Arduino's USB power (5V, very low current). This cannot hurt you. Do NOT use this while also connected to mains-powered audio equipment. Keep it simple: just USB to laptop.

Step 1: Stack the Muscle BioAmp Shield onto the Arduino Take the Arduino Uno R4 out of its box and lay it flat on a table. Take the Muscle BioAmp Shield v0.3, the small green circuit board. Look at the bottom of the shield: you'll see two rows of metal pins. Look at the top of the Arduino: you'll see matching rows of black sockets. Line up the pins with the sockets VERY carefully. All pins must align before you push down. Press the shield down FIRMLY and evenly with both thumbs until you feel it click into place. Result: one combined board, the shield sitting on top of the Arduino.

What does the shield look like when stacked correctly? The shield should sit flat and level on top of the Arduino. No pins should be sticking out to the side. The USB-C port of the Arduino should still be fully accessible from the end.

Step 2: Prepare your skin Get the skin wipes and NuPrep Gel from your kit. Find the spot on the back of your neck / upper shoulder: specifically the trapezius muscle. It's the fleshy area that runs from the base of your skull outward to your shoulder. Squeeze your shoulder up toward your ear and you'll feel it tighten. That's the muscle. Wipe the skin on that area with a wipe to remove any oil or lotion. Let it dry for 30 seconds. Put a tiny pea-sized blob of NuPrep Gel on the tip of your finger. Rub the gel on the skin where the electrodes will go, using a slight circular rubbing motion for about 10 seconds. Wipe the gel off with a clean tissue. The skin might look slightly pink. That's normal and means the skin is prepared. Why? NuPrep gel gently removes the outer dead skin layer, which is a poor electrical conductor. It dramatically improves your signal quality.

Step 3: Place the three gel electrodes Take 3 gel electrodes from the pack. They look like small round sticky pads with a metal snap button in the middle. You will place them in a triangular arrangement on your upper trapezius:

| Electrode | Where to place it |
| --- | --- |
| IN+ (Positive) | On the belly of the right trapezius muscle, about 3 cm from the base of your neck |
| IN- (Negative) | On the same muscle, about 2 cm to the right of the first electrode |
| REF (Reference) | On a bony area with no muscle, like the back of your elbow bone or your wrist bone |

Peel the backing off each electrode and press it firmly onto the skin. Press around the edges to make sure it's stuck properly. The reference electrode (REF) can go on the bony part of your elbow, or anywhere with no muscle underneath.

Why 3 electrodes? IN+ and IN- measure the difference in electrical potential between two points on the muscle. The signal between them is the actual muscle activity. REF (Reference) is a ground point. It cancels out electrical noise from the environment, like interference from nearby power cables. Without it, your signal would be very noisy and hard to read.

Step 4: Connect the BioAmp Cable Find the BioAmp Cable (100cm). It has 3 coloured snap connectors on one end and a small plug on the other. Snap each connector onto the matching electrode: RED to IN+, BLACK to IN-, GREEN to REF. (Check the colour labels on your specific cable.) Plug the other end of the cable into the 3-pin header socket on the Muscle BioAmp Shield. The socket is clearly labelled IN+, IN-, REF on the board. Make sure the cable is not tugging on the electrodes: run it over your shoulder so it doesn't pull.

Step 5: Connect Arduino to your laptop Find the USB-C cable from your kit. Plug the USB-C end into the Arduino Uno R4. Plug the USB-A end into your laptop. The green power LED on the Arduino should light up. If it doesn't, try a different USB port. You're now powering the Arduino from your laptop, no batteries needed!

# Section 3: Setting Up the Arduino

Now we need to upload the sketch to the Arduino. It reads the muscle sensor every 2 milliseconds, spends the first 4 seconds calibrating a resting baseline, computes three rolling RMS windows, blends them into a posture score, and streams the results to the laptop as CSV. The onboard LED also blinks during calibration and flashes during an alert, so you can see status without even looking at the laptop.

Step 1: Download and install the Arduino IDE Open your web browser on your laptop. Go to: arduino.cc/en/software Click 'Download Arduino IDE 2' and choose the version for your operating system (Windows, Mac, or Linux). Open the downloaded file and follow the installer. Click 'Next' on everything. The default settings are fine. When it's done, open Arduino IDE. You'll see a white window with some code already in it.

Step 2: Select your Arduino board In Arduino IDE, go to the top menu: Tools > Board > Arduino AVR Boards OR Arduino UNO R4 Boards. Select 'Arduino Uno R4 WiFi' (that's what's in your kit). Now go to: Tools > Port. You'll see a list of ports. Select the one that says something like: COM3 (Windows) or /dev/ttyACM0 (Mac/Linux). If you're not sure which port is the Arduino, unplug the USB cable, look at the list, plug it back in, and see which new one appears. That's your Arduino!

How to find your COM port on Windows Click the Start button and type: Device Manager Open it and look for 'Ports (COM & LPT)' Click the arrow to expand it You should see something like: Arduino Uno (COM3). That's your port number!

Step 3: Type in the Arduino sketch In Arduino IDE, delete everything in the white area (select all with Ctrl+A, then Delete). This sketch is longer than a beginner 'hello world', so copy-pasting it (rather than retyping by hand) is strongly recommended if you're reading this digitally.

// ════════════════════════════════════════════════════════════════ // NECK-BEND POSTURE MONITOR — Arduino UNO R4 Minima (v6.0) // Muscle BioAmp Shield v0.3 on A0 // Electrodes: IN+/IN- on UPPER TRAPEZIUS (2 cm apart, either side) // REF on bony point (back of elbow / wrist bone) // // v6.0 CHANGES vs v5.0: // ───────────────────────────────────────────────────────────── // • BALANCED sensitivity: SENS_LONG_X10 11→14, SENS_TREND_X10 10→13, // SCORE_THRESHOLD 20→28. v5.0 fired on swallows/talking/shifting; // this catches real sustained forward-neck tension while ignoring // momentary twitches. // • NEW: debounced alert gate (DEBOUNCE_SAMPLES=40, ~84ms). The old // code re-evaluated longAbove/trendAbove every single 2ms sample // with no hysteresis, so noise near the threshold line flickered // neckBendFlag 0/1/0/1 rapidly — which on the host side kept // resetting the ALERT_HOLD_SEC timer and made alarms unreliable. // Now the flag only flips after 40 consecutive samples agree. // • Fixed rounding in printed THRESH_* calibration values (cosmetic // only — did not affect detection math, which already used the // full-precision baselineX10). // // HOW NECK-BEND IS DETECTED: // ───────────────────────────────────────────────────────────── // Forward neck bend loads the upper trapezius to hold the ~5 kg head. // Three RMS windows measure this: // 1. SHORT-RMS (100 ms) — fast spikes — score weight only // 2. LONG-RMS (300 ms) — sustained tension — primary gate // 3. TREND-RMS (1.2 s) — progressive slouch — secondary gate // // ALERT fires when: // postureScore >= SCORE_THRESHOLD (28) // AND ( longRMS > 1.4× baseline OR trendRMS > 1.3× baseline ) // AND that condition holds for 40 consecutive samples (~84 ms) // // CALIBRATION (4 seconds on power-up): // → Keep neck STRAIGHT and muscles FULLY RELAXED // → LED blinks fast during calibration // → LED solid ON = calibration done, monitoring started // // Serial CSV at ~476 Hz: // raw, rmsShort, rmsLong, rmsTrend, postureScore, neckBendFlag // ════════════════════════════════════════════════════════════════ #define EMG_PIN A0 #define LED_PIN 13 #define SAMPLE_DELAY_MS 2 // ~476 Hz // Window sizes in samples #define WIN_SHORT 50 // 100 ms #define WIN_LONG 150 // 300 ms #define WIN_TREND 600 // 1200 ms #define CALIB_SECS 4 #define FLOOR_RMS 5 // ── Sensitivity multipliers (×10 fixed-point) ──────────────────── // BALANCED tuning: catches real sustained forward-neck bends, // rides through small twitches (swallowing, talking, shifting). #define SENS_SHORT_X10 16 // 1.6× baseline — score weight only #define SENS_LONG_X10 14 // 1.4× baseline — alert gate #define SENS_TREND_X10 13 // 1.3× baseline — slow slouch gate // Score threshold — lower = more sensitive #define SCORE_THRESHOLD 28 // ── Alert debounce ──────────────────────────────────────────────── // Requires N consecutive over-threshold samples before flag goes HIGH, // and N consecutive under-threshold samples before it drops back to LOW. // At ~476 Hz, 40 samples ≈ 84 ms — kills single-sample noise flicker // without adding noticeable lag to genuine bends. #define DEBOUNCE_SAMPLES 40 // ── Circular buffers ───────────────────────────────────────────── int shortBuf[WIN_SHORT]; int longBuf [WIN_LONG ]; int trendBuf[WIN_TREND]; int shortIdx = 0, longIdx = 0, trendIdx = 0; bool shortFull = false, longFull = false, trendFull = false; long baselineX10 = FLOOR_RMS * 10L; bool calibDone = false; int debounceCounter = 0; // + when over threshold, - when under bool alertActive = false; // ── Integer square root ────────────────────────────────────────── unsigned long isqrt(unsigned long n) { if (n == 0) return 0; unsigned long x = n, y = (x + 1) >> 1; while (y < x) { x = y; y = (x + n / x) >> 1; } return x; } // ── Compute RMS centred on ADC midpoint 512 ────────────────────── unsigned long computeRMS(int* buf, int len) { unsigned long sumSq = 0; for (int i = 0; i < len; i++) { long v = (long)buf[i] - 512L; sumSq += (unsigned long)(v * v); } return isqrt(sumSq / (unsigned long)len); } void setup() { Serial.begin(115200); unsigned long t0 = millis(); while (!Serial && millis() - t0 < 3000) { ; } pinMode(LED_PIN, OUTPUT); // Pre-fill with midpoint so RMS starts at zero for (int i = 0; i < WIN_SHORT; i++) shortBuf[i] = 512; for (int i = 0; i < WIN_LONG; i++) longBuf[i] = 512; for (int i = 0; i < WIN_TREND; i++) trendBuf[i] = 512; Serial.println("CALIBRATING"); // Calibration: measure resting RMS — keep neck straight, muscles relaxed unsigned long sumSq = 0, count = 0; unsigned long calEnd = millis() + (unsigned long)CALIB_SECS * 1000UL; while (millis() < calEnd) { int v = analogRead(EMG_PIN) - 512; sumSq += (unsigned long)((long)v * v); count++; digitalWrite(LED_PIN, (millis() / 200) & 1); delay(SAMPLE_DELAY_MS); } unsigned long meanSq = (count > 0) ? (sumSq / count) : 1UL; unsigned long bRMS = isqrt(meanSq); if (bRMS < (unsigned long)FLOOR_RMS) bRMS = FLOOR_RMS; baselineX10 = (long)bRMS * 10L; digitalWrite(LED_PIN, HIGH); calibDone = true; Serial.print("BASELINE:"); Serial.println(bRMS); Serial.print("THRESH_SHORT:"); Serial.println((bRMS * SENS_SHORT_X10 + 5) / 10); Serial.print("THRESH_LONG:"); Serial.println((bRMS * SENS_LONG_X10 + 5) / 10); Serial.print("THRESH_TREND:"); Serial.println((bRMS * SENS_TREND_X10 + 5) / 10); } void loop() { int raw = analogRead(EMG_PIN); shortBuf[shortIdx++] = raw; if (shortIdx >= WIN_SHORT) { shortIdx = 0; shortFull = true; } longBuf[longIdx++] = raw; if (longIdx >= WIN_LONG) { longIdx = 0; longFull = true; } trendBuf[trendIdx++] = raw; if (trendIdx >= WIN_TREND) { trendIdx = 0; trendFull = true; } unsigned long rmsS = shortFull ? computeRMS(shortBuf, WIN_SHORT) : 0UL; unsigned long rmsL = longFull ? computeRMS(longBuf, WIN_LONG) : 0UL; unsigned long rmsT = trendFull ? computeRMS(trendBuf, WIN_TREND) : 0UL; int neckBendFlag = 0; int postureScore = 0; if (calibDone && shortFull && longFull) { long scoreS = ((long)rmsS * 100L * 10L) / baselineX10 - 100L; long scoreL = ((long)rmsL * 100L * 10L) / baselineX10 - 100L; long scoreT = trendFull ? (((long)rmsT * 100L * 10L) / baselineX10 - 100L) : 0L; if (scoreS < 0) scoreS = 0; if (scoreL < 0) scoreL = 0; if (scoreT < 0) scoreT = 0; // Weighted blend: LONG dominates (sustained tension) postureScore = (int)((scoreS * 20L + scoreL * 65L + scoreT * 15L) / 100L); if (postureScore > 100) postureScore = 100; // Alert gates: 1.4× baseline for LONG, 1.3× for TREND bool longAbove = ((long)rmsL * 100L > baselineX10 * (long)SENS_LONG_X10); bool trendAbove = trendFull && ((long)rmsT * 100L > baselineX10 * (long)SENS_TREND_X10); bool rawOver = (postureScore >= SCORE_THRESHOLD) && (longAbove ¦¦ trendAbove); // Debounce: require DEBOUNCE_SAMPLES consecutive samples in the new // state before actually flipping the flag. Stops single-sample noise // from toggling the alert and resetting the host-side hold timer. if (rawOver) { debounceCounter = (debounceCounter < 0) ? 1 : debounceCounter + 1; } else { debounceCounter = (debounceCounter > 0) ? -1 : debounceCounter - 1; } // Clamp so it can't climb indefinitely during a long sustained bend // or a long good-posture stretch — only the threshold crossing matters. if (debounceCounter > DEBOUNCE_SAMPLES) debounceCounter = DEBOUNCE_SAMPLES; if (debounceCounter < -DEBOUNCE_SAMPLES) debounceCounter = -DEBOUNCE_SAMPLES; if (debounceCounter >= DEBOUNCE_SAMPLES) alertActive = true; if (debounceCounter <= -DEBOUNCE_SAMPLES) alertActive = false; if (alertActive) { neckBendFlag = 1; digitalWrite(LED_PIN, (millis() / 100) & 1); // fast blink = alert } else { digitalWrite(LED_PIN, HIGH); // solid = good posture } } // CSV: raw, rmsShort, rmsLong, rmsTrend, postureScore, neckBendFlag Serial.print(raw); Serial.print(','); Serial.print(rmsS); Serial.print(','); Serial.print(rmsL); Serial.print(','); Serial.print(rmsT); Serial.print(','); Serial.print(postureScore); Serial.print(','); Serial.println(neckBendFlag); delay(SAMPLE_DELAY_MS); }

What this code does: On power-up it spends 4 seconds calibrating. Keep your neck straight and muscles relaxed while the LED blinks fast. After that the LED goes solid and monitoring begins. About 476 times a second it reads the muscle sensor, updates three rolling RMS windows (100ms, 300ms, and 1.2s), and blends them into a 0 to 100 posture score. That score has to stay over threshold for 40 consecutive samples (~84ms) before the neckBendFlag actually goes high. This debounce stops a single noisy sample from triggering a false alert. Every sample is streamed to the laptop as one CSV line: raw, rmsShort, rmsLong, rmsTrend, postureScore, neckBendFlag.

Step 4: Upload the code to Arduino Click the right-pointing arrow button at the top of Arduino IDE (the Upload button). It looks like: --> You'll see 'Compiling sketch...' at the bottom. Wait for this. Then you'll see 'Uploading...' and the orange lights on your Arduino will flicker. Finally you'll see: 'Done uploading.' in green text at the bottom. If you see an error: double-check that the correct COM port is selected under Tools > Port.

## Quick Test: Verify the signal is working

Before moving on, let's check that the Arduino is sending data:

- In Arduino IDE, click: Tools > Serial Monitor

- Set the baud rate dropdown at the bottom right to: 115200

- You'll first see the word CALIBRATING, then three THRESH_ lines once the 4-second calibration finishes. After that, a stream of comma-separated numbers, one line per sample: raw, rmsShort, rmsLong, rmsTrend, postureScore, neckBendFlag (e.g. 512,3,4,3,0,0).

- Sit still and relaxed while it calibrates. Once it's done, clench your upper shoulder muscle (the trapezius). The raw value and the RMS columns should jump up, and postureScore should climb.

- Relax. The numbers should settle back down, and the last column (neckBendFlag) should read 0 again.

If you see numbers changing, great! Your hardware is working perfectly. Close the Serial Monitor (we'll need the port free for Python next).

# Section 4: Setting Up Python on Your Laptop

Python is a free programming language that runs on your laptop. We'll use it to read the Arduino data, process it, and show you alerts and graphs.

Step 1: Install Python Open your web browser and go to: python.org/downloads Click the big yellow 'Download Python 3.x.x' button (any version starting with 3 is fine). Open the downloaded file. IMPORTANT: On the first installer screen, tick the checkbox that says 'Add Python to PATH'. This is easy to miss! Then click 'Install Now' and wait for it to finish.

What is PATH? Think of PATH like a phone book that tells your computer where to find programs. If Python is in the PATH, you can run it from anywhere by just typing 'python'. If it's not, you'd have to type the full address every time, which gets annoying fast!

Step 2: Open the Terminal / Command Prompt Windows: Press the Windows key, type 'cmd', press Enter. A black window appears. Mac: Press Cmd+Space, type 'Terminal', press Enter. A white window appears. Linux: Press Ctrl+Alt+T. This is the terminal: you type commands here. Don't worry, we'll tell you exactly what to type!

Step 3: Install the required Python libraries In the terminal, type this command exactly and press Enter:

pip install pyserial numpy matplotlib sounddevice

You'll see a lot of text scrolling by. That's normal. Wait for it to finish (about 1-2 minutes). You should see 'Successfully installed...' at the end.

| Library name | What it does |
| --- | --- |
| pyserial | Lets Python talk to the Arduino over USB, reads the CSV stream |
| numpy | Maths library, used for the RMS/score maths and to synthesize the alarm beep |
| sounddevice | Plays the two-tone alarm beep through your speakers. If it's not installed, the script automatically falls back to winsound (built into Windows), then pygame, then a terminal beep |
| matplotlib | Draws the live cyberpunk dashboard, the EMG trace and posture-score graph |

Step 4: Create a folder for your project On your laptop, create a new folder somewhere easy to find, for example on your Desktop. Call it: PostureMonitor We'll save our Python script inside this folder.

# Section 5: The NeckWatch Python Script

This is the main program that turns all the hardware into your live posture dashboard. It's a bigger script than a beginner 'hello world', so we'll walk through what each part does, then give you the full file to copy in one go.

## 5.1  Understanding the script in plain English

Here's what the script does, step by step:

- Auto-detects the Arduino's COM port (or uses one you set manually), and retries a few times if the port is busy

- Reads each CSV line from the Arduino and keeps a rolling history of the raw signal and posture score for the on-screen graphs

- Tracks how long the posture score has stayed above the alert threshold (SCORE_ALARM), and fires an alert once it's been held for ALERT_HOLD_SEC seconds

- Plays a two-tone alarm beep (via sounddevice, winsound, or pygame, whichever is available) and enforces a cooldown so it doesn't fire again immediately

- Draws a live two-panel dashboard (raw EMG on top, posture score history below) with a session timer, peak-score readout, and alert counter

## 5.2  The full script: save this as posture_monitor.py

""" ╔══════════════════════════════════════════════════════════════════════╗ ║ NECK-BEND POSTURE MONITOR · v8.0 · Cyberpunk Live Dashboard ║ ║ Arduino UNO R4 Minima + Muscle BioAmp Shield v0.3 ║ ╠══════════════════════════════════════════════════════════════════════╣ ║ v8.0 — bigger, faster, more cyberpunk: ║ ║ · Scroll-back window 10s → 25s and score history 300 → 900 pts, ║ ║ so both panels show a lot more context before scrolling off. ║ ║ · Redraw rate 25fps → 50fps (FPS_MS 40→20) for smoother motion. ║ ║ · Score panel now has a real time axis (seconds) like the EMG one.║ ║ · New HUD chrome: animated CRT scanline sweep, corner brackets on ║ ║ both panels, live session-timer readout, live PEAK score ║ ║ readout, and a subtle glitch-jitter on the alarm banner. ║ ║ · v7.0 behavior kept: live EMG + posture score + beep alarm only. ║ ║ · Sensitivity: SCORE_ALARM=20, ALERT_HOLD_SEC=0.6s (unchanged). ║ ║ · Fixes "Access is denied" on the COM port: that error means ║ ║ something else already has it open (Arduino Serial Monitor, or ║ ║ a previous run of this script that didn't exit cleanly). The ║ ║ script now retries a few times with a clear message instead of ║ ║ dying instantly, and always closes the port on exit. ║ ╚══════════════════════════════════════════════════════════════════════╝ """ import sys, os, time, threading, io, wave, tempfile, traceback from collections import deque try: import numpy as np import serial import serial.tools.list_ports import matplotlib import matplotlib.pyplot as plt import matplotlib.animation as animation import matplotlib.patheffects as pe import matplotlib.font_manager as fm except ImportError as e: print(f"\n[MISSING LIBRARY] {e}") print("Run: pip install pyserial numpy matplotlib\n") sys.exit(1) # ── Cyberpunk-style fonts (Orbitron for big display text, Rajdhani for HUD ── # ── body text). Both are free/open-license Google Fonts, not the actual ── # ── in-game CD Projekt Red font (which isn't freely redistributable). ── FONT_DIR = os.path.join(os.path.dirname(os.path.abspath(__file__)), "fonts") DISPLAY_FONT = "monospace" # big glowing headers / SCORE / alarm banner BODY_FONT = "monospace" # axis labels, ticks, HUD readouts try: for _fname in ("Orbitron.ttf", "Rajdhani-Regular.ttf", "Rajdhani-Bold.ttf", "Rajdhani-SemiBold.ttf"): _fpath = os.path.join(FONT_DIR, _fname) if os.path.isfile(_fpath): fm.fontManager.addfont(_fpath) DISPLAY_FONT = fm.FontProperties(fname=os.path.join(FONT_DIR, "Orbitron.ttf")).get_name() BODY_FONT = fm.FontProperties(fname=os.path.join(FONT_DIR, "Rajdhani-SemiBold.ttf")).get_name() print(f"[FONT] Cyberpunk theme loaded: display='{DISPLAY_FONT}' body='{BODY_FONT}'") except Exception as e: print(f"[FONT] Couldn't load custom fonts ({e}) — falling back to monospace. " f"Make sure the 'fonts' folder sits next to this script.") # ── Sound backend: sounddevice → winsound → pygame → terminal beep ─── SND = "none" try: import sounddevice as sd SND = "sounddevice" print("[SOUND] Backend: sounddevice ✓") except (ImportError, OSError): try: import winsound SND = "winsound" print("[SOUND] Backend: winsound ✓") except ImportError: try: import pygame pygame.mixer.init(frequency=44100, size=-16, channels=1, buffer=512) SND = "pygame" print("[SOUND] Backend: pygame ✓") except Exception: SND = "beep" print("[SOUND] Backend: terminal beep (install sounddevice for real audio)") # ══════════════════════════════════════════════════════════════════════ # CONFIG — sensitivity raised vs v6.0 # ══════════════════════════════════════════════════════════════════════ PORT = None # None = auto-detect, or set e.g. "COM11" BAUD = 115_200 SCORE_ALARM = 20 # was 28 — fires sooner now ALERT_HOLD_SEC = 0.6 # was 1.0 — less time bent before alarm fires ALERT_COOLDOWN = 6 # was 8 — alarms can repeat a bit sooner RECONNECT_DELAY_SEC = 2.0 PORT_OPEN_RETRIES = 5 # retries on "Access is denied" / busy port before giving up PORT_OPEN_RETRY_WAIT = 1.5 DISPLAY_SECS = 25 # was 10 — much longer EMG scroll-back window SAMPLE_RATE = 476 # matches Arduino SAMPLE_DELAY_MS=2 (~476Hz) — only change if the .ino changes too DISPLAY_LEN = DISPLAY_SECS * SAMPLE_RATE RAW_DECIMATE = 5 # was 4 — slightly higher so the longer window still draws fast FPS_MS = 20 # was 40 — 50fps redraw instead of 25fps, noticeably smoother SCORE_HISTORY = 900 # was 300 — ~3x more posture-score history retained on screen # ══════════════════════════════════════════════════════════════════════ # PORT HANDLING # ══════════════════════════════════════════════════════════════════════ def find_arduino_port(): candidates = [] for p in serial.tools.list_ports.comports(): desc, mfr, vid = (p.description or "").lower(), (p.manufacturer or "").lower(), p.vid if vid in (0x2341, 0x1A86, 0x0403): candidates.insert(0, p.device) elif any(k in desc or k in mfr for k in ("arduino", "ch340", "cp210", "ftdi", "uno")): candidates.append(p.device) elif p.device.startswith(("/dev/ttyACM", "/dev/ttyUSB", "COM")): candidates.append(p.device) return candidates[0] if candidates else None def open_port(port, baud): """Open the serial port, retrying on 'busy/access denied' instead of dying instantly. That error means another program (Arduino IDE's Serial Monitor/Plotter, or a previous unclosed run of this script) already has the port open.""" last_err = None for attempt in range(1, PORT_OPEN_RETRIES + 1): try: return serial.Serial(port, baud, timeout=1) except serial.SerialException as e: last_err = e msg = str(e).lower() if "access is denied" in msg or "permissionerror" in msg or "could not open port" in msg: print(f"[RETRY {attempt}/{PORT_OPEN_RETRIES}] {port} is busy — " f"close Arduino IDE's Serial Monitor/Plotter and any other " f"running copy of this script. Retrying in {PORT_OPEN_RETRY_WAIT}s…") else: print(f"[RETRY {attempt}/{PORT_OPEN_RETRIES}] {e}") time.sleep(PORT_OPEN_RETRY_WAIT) raise last_err # ══════════════════════════════════════════════════════════════════════ # CYBERPUNK PALETTE # ══════════════════════════════════════════════════════════════════════ BG, PANEL, BORDER, GRID = "#030508", "#080D18", "#152A4A", "#0C1828" NEON_CYAN, NEON_PINK, NEON_GREEN = "#00E5FF", "#FF1AAD", "#00FF88" NEON_AMBER, NEON_RED = "#FFBE00", "#FF1744" TEXT, DIM = "#D0F0FF", "#2A4A6A" def glow(color, lw=7, alpha=0.28): return [pe.SimpleLineShadow(offset=(0, 0), shadow_color=color, alpha=alpha, linewidth=lw), pe.Normal()] # ══════════════════════════════════════════════════════════════════════ # ALARM SOUND — simple ascending two-tone beep # ══════════════════════════════════════════════════════════════════════ SR = 44100 def _synth_beep(freq, duration_ms=220, vol=0.8): n = int(SR * duration_ms / 1000) t = np.linspace(0, duration_ms / 1000, n, endpoint=False) sig = np.sin(2 * np.pi * freq * t) env = np.ones(n) atk, rel = int(n * 0.05), int(n * 0.3) env[:atk] = np.linspace(0, 1, atk) env[-rel:] = np.linspace(1, 0, rel) return (sig * env * vol).astype(np.float32) _ALARM_FREQS = [880, 1100, 880, 1100] _ALARM_DURS = [180, 180, 180, 220] _WAV_FILES, _PYG_SOUNDS = {}, {} def _prepare_sounds(): for f, d in zip(_ALARM_FREQS, _ALARM_DURS): if f in _WAV_FILES: continue arr = _synth_beep(f, d) if SND in ("winsound", "pygame"): pcm = (arr * 32767).astype(np.int16).tobytes() out = io.BytesIO() with wave.open(out, "wb") as wf: wf.setnchannels(1); wf.setsampwidth(2); wf.setframerate(SR); wf.writeframes(pcm) if SND == "winsound": tmp = tempfile.NamedTemporaryFile(suffix=".wav", delete=False) tmp.write(out.getvalue()); tmp.close() _WAV_FILES[f] = tmp.name else: out.seek(0) _PYG_SOUNDS[f] = pygame.mixer.Sound(out) _prepare_sounds() def play_alarm(): def _go(): for freq, dur in zip(_ALARM_FREQS, _ALARM_DURS): try: if SND == "sounddevice": sd.play(_synth_beep(freq, dur), SR); sd.wait(); time.sleep(0.02) elif SND == "winsound": winsound.PlaySound(_WAV_FILES.get(freq), winsound.SND_FILENAME ¦ winsound.SND_NOSTOP) time.sleep(dur / 1000 + 0.03) elif SND == "pygame": s = _PYG_SOUNDS.get(freq) if s: s.play() time.sleep(dur / 1000 + 0.03) else: print("\a", end="", flush=True); time.sleep(0.25) except Exception as ex: print(f"[SND] {ex}") threading.Thread(target=_go, daemon=True).start() # ══════════════════════════════════════════════════════════════════════ # SHARED STATE # ══════════════════════════════════════════════════════════════════════ raw_buf = deque([512] * DISPLAY_LEN, maxlen=DISPLAY_LEN) score_buf = deque([0] * SCORE_HISTORY, maxlen=SCORE_HISTORY) peak_score = 0 baseline_rms = None thresh_long = None last_alert_t = 0.0 alert_flash = 0 session_alerts = 0 session_start = time.time() latest_score = 0 latest_bend = 0 status_text, status_col = "Waiting for Arduino…", DIM calibrating = True _bad_hold_start = None ser, ser_lock = None, threading.Lock() ser_queue = deque(maxlen=4000) last_data_t = [time.time()] _frame_error_logged = False # ══════════════════════════════════════════════════════════════════════ # SERIAL READER THREAD # ══════════════════════════════════════════════════════════════════════ def serial_reader(): global calibrating, baseline_rms, thresh_long, status_text, status_col, ser while True: try: line = ser.readline().decode("utf-8", errors="ignore").strip() if not line: continue if line == "CALIBRATING": status_text, status_col, calibrating = "CALIBRATING — sit STRAIGHT, muscles RELAXED…", NEON_AMBER, True continue if line.startswith("BASELINE:"): try: baseline_rms = float(line.split(":")[1]) status_text, status_col, calibrating = ( f"CALIBRATED · Baseline={baseline_rms:.1f} · DETECTION ACTIVE", NEON_GREEN, False) except ValueError: pass continue if line.startswith("THRESH_LONG:"): try: thresh_long = float(line.split(":")[1]) except ValueError: pass continue if line.startswith("THRESH_SHORT:") or line.startswith("THRESH_TREND:"): continue # not displayed in the trimmed UI with ser_lock: ser_queue.append(line) last_data_t[0] = time.time() except (serial.SerialException, OSError): status_text, status_col = "SERIAL DISCONNECTED — reconnecting…", NEON_RED try: if ser is not None: ser.close() except Exception: pass time.sleep(RECONNECT_DELAY_SEC) try: target_port = PORT or find_arduino_port() if target_port: ser = open_port(target_port, BAUD) status_text, status_col, calibrating = f"RECONNECTED on {target_port} — recalibrating…", NEON_AMBER, True else: status_text = "SERIAL DISCONNECTED — no Arduino port found, retrying…" except Exception: pass except Exception: time.sleep(0.05) # ══════════════════════════════════════════════════════════════════════ # ALERT LOGIC # ══════════════════════════════════════════════════════════════════════ def fire_alert(score): global last_alert_t, alert_flash, session_alerts now = time.time() if now - last_alert_t < ALERT_COOLDOWN: return last_alert_t, session_alerts = now, session_alerts + 1 alert_flash = int(3.0 * 1000 / FPS_MS) ts = time.strftime("%H:%M:%S") print(f"\n{'█'*56}\n ⚠ NECK BEND ALERT [{ts}] Score={score}\n → Lift your gaze · roll shoulders back · sit tall\n{'█'*56}\n") play_alarm() # ══════════════════════════════════════════════════════════════════════ # BUILD FIGURE — two panels: raw EMG, posture score # ══════════════════════════════════════════════════════════════════════ matplotlib.rcParams.update({ "font.family": BODY_FONT, "text.color": TEXT, "axes.labelcolor": DIM, "xtick.color": DIM, "ytick.color": DIM, "axes.edgecolor": BORDER, "axes.facecolor": PANEL, "figure.facecolor": BG, "axes.grid": True, "grid.color": GRID, "grid.linewidth": 0.4, "axes.spines.top": False, "axes.spines.right": False, "xtick.labelsize": 8, "ytick.labelsize": 8, }) fig, (ax_raw, ax_score) = plt.subplots(2, 1, figsize=(11, 7), facecolor=BG, gridspec_kw={"height_ratios": [1.1, 1], "hspace": 0.55}) fig.canvas.manager.set_window_title("POSTURE MONITOR v8.0") plt.subplots_adjust(left=0.08, right=0.96, top=0.86, bottom=0.08) for ax in (ax_raw, ax_score): for sp in ax.spines.values(): sp.set_color(BORDER); sp.set_linewidth(0.8) fig.text(0.5, 0.965, "// NEURAL POSTURE MONITOR // LIVE EMG //", ha="center", va="top", fontsize=15, fontweight="bold", color=NEON_CYAN, fontfamily=DISPLAY_FONT, path_effects=glow(NEON_CYAN, 9, 0.32)) status_obj = fig.text(0.5, 0.92, status_text, ha="center", va="top", fontsize=9.5, color=status_col, fontfamily=BODY_FONT, fontweight="bold") # ── Raw EMG panel ── ax_raw.set_title("RAW EMG · UPPER TRAPEZIUS (forward neck -> ^ activity)", color=NEON_CYAN, fontsize=10, fontweight="bold", loc="left", pad=4, fontfamily=BODY_FONT) ax_raw.set_ylabel("ADC", fontsize=8, fontfamily=BODY_FONT) ax_raw.set_ylim(0, 1023) ax_raw.set_xlim(0, DISPLAY_LEN) ax_raw.xaxis.set_major_formatter(matplotlib.ticker.FuncFormatter(lambda x, _: f"{x/SAMPLE_RATE:.0f}s")) x_raw_arr = np.arange(DISPLAY_LEN) x_raw_dec = x_raw_arr[::RAW_DECIMATE] line_raw, = ax_raw.plot([], [], lw=0.9, color=NEON_CYAN, alpha=0.95, path_effects=glow(NEON_CYAN, 5, 0.22)) thresh_l_line = ax_raw.axhline(0, color=NEON_RED, lw=0.8, ls="--", alpha=0) live_score_txt = ax_raw.text(0.99, 0.92, "SCORE: —", transform=ax_raw.transAxes, ha="right", va="top", fontsize=13, fontweight="bold", color=NEON_GREEN, fontfamily=DISPLAY_FONT) live_bend_txt = ax_raw.text(0.99, 0.72, "FLAG: —", transform=ax_raw.transAxes, ha="right", va="top", fontsize=10, color=DIM, fontfamily=BODY_FONT, fontweight="bold") # ── Score panel ── ax_score.set_title(f"POSTURE SCORE · alert ≥ {SCORE_ALARM} held {ALERT_HOLD_SEC}s", color=NEON_PINK, fontsize=10, fontweight="bold", loc="left", pad=4, fontfamily=BODY_FONT) ax_score.set_ylabel("Score", fontsize=8, fontfamily=BODY_FONT) ax_score.set_ylim(0, 105) ax_score.set_xlim(0, SCORE_HISTORY) ax_score.xaxis.set_major_formatter(matplotlib.ticker.FuncFormatter(lambda x, _: f"{x/SAMPLE_RATE:.0f}s")) ax_score.axhspan(0, SCORE_ALARM, alpha=0.08, color=NEON_GREEN) ax_score.axhspan(SCORE_ALARM, 65, alpha=0.08, color=NEON_AMBER) ax_score.axhspan(65, 105, alpha=0.08, color=NEON_RED) ax_score.axhline(SCORE_ALARM, color=NEON_AMBER, lw=0.9, ls="--", alpha=0.75) line_score, = ax_score.plot([], [], lw=2.0, color=NEON_PINK, path_effects=glow(NEON_PINK)) hold_txt = ax_score.text(0.99, 0.92, "", transform=ax_score.transAxes, ha="right", va="top", fontsize=10, color=NEON_AMBER, fontfamily=BODY_FONT, fontweight="bold") alerts_txt = ax_score.text(0.01, 0.92, "ALERTS: 0", transform=ax_score.transAxes, ha="left", va="top", fontsize=9, color=DIM, fontfamily=BODY_FONT, fontweight="bold") peak_score_txt = ax_score.text(0.01, 0.06, "PEAK: 0", transform=ax_score.transAxes, ha="left", va="bottom", fontsize=9, color=DIM, fontfamily=BODY_FONT, fontweight="bold") # ── Full-screen alarm flash overlay ── (disabled: banner text removed) alarm_scrim = fig.add_axes([0, 0, 1, 1], zorder=10) alarm_scrim.axis("off") alarm_scrim.set_facecolor(NEON_RED); alarm_scrim.patch.set_alpha(0) alarm_title = fig.text(0.5, 0.5, "", ha="center", va="center", fontsize=30, fontweight="bold", color=NEON_RED, alpha=0, fontfamily=DISPLAY_FONT, zorder=11, path_effects=glow(NEON_RED, 12, 0.5)) alarm_sub = fig.text(0.5, 0.43, "", ha="center", va="center", fontsize=13, color=TEXT, alpha=0, fontfamily=BODY_FONT, fontweight="bold", zorder=11) # ── HUD corner brackets — static cyberpunk chrome on each panel ── def _corner_brackets(ax, size=0.045, color=NEON_CYAN, lw=1.3, alpha=0.55): for (cx, cy, dx, dy) in [(0, 0, 1, 1), (1, 0, -1, 1), (0, 1, 1, -1), (1, 1, -1, -1)]: ax.plot([cx, cx + dx * size], [cy, cy], transform=ax.transAxes, color=color, lw=lw, alpha=alpha, clip_on=False, solid_capstyle="round") ax.plot([cx, cx], [cy, cy + dy * size], transform=ax.transAxes, color=color, lw=lw, alpha=alpha, clip_on=False, solid_capstyle="round") _corner_brackets(ax_raw, color=NEON_CYAN) _corner_brackets(ax_score, color=NEON_PINK) # ── Animated CRT scanline sweep — a faint band drifting down the whole window ── scan_ax = fig.add_axes([0, 0, 1, 1], zorder=9) scan_ax.axis("off") scan_ax.set_facecolor("none") scan_rect = plt.Rectangle((0, 1.0), 1, 0.006, transform=scan_ax.transAxes, color=NEON_CYAN, alpha=0.10, lw=0) scan_ax.add_patch(scan_rect) _scan_y = [1.0] # ── Session timer readout ── session_timer_txt = fig.text(0.965, 0.965, "T+00:00:00", ha="right", va="top", fontsize=10, color=NEON_CYAN, fontfamily=DISPLAY_FONT, alpha=0.85) _ANIMATED_ARTISTS = [line_raw, line_score, thresh_l_line, live_score_txt, live_bend_txt, hold_txt, alerts_txt, peak_score_txt, status_obj, alarm_scrim.patch, alarm_title, alarm_sub, scan_rect, session_timer_txt] _ALARM_ARTISTS = [alarm_title, alarm_sub] # ══════════════════════════════════════════════════════════════════════ # FRAME UPDATE # ══════════════════════════════════════════════════════════════════════ def _update_impl(frame): global alert_flash, _bad_hold_start, latest_score, latest_bend, peak_score status_obj.set_text(status_text) status_obj.set_color(status_col) with ser_lock: lines = list(ser_queue) ser_queue.clear() for line in lines[-200:]: parts = line.split(",") if len(parts) != 6: continue try: raw, rmsS, rmsL, rmsT, score, bend = (int(float(p)) for p in parts) except ValueError: continue raw_buf.append(raw) score_buf.append(score) latest_score, latest_bend = score, bend # raw EMG trace arr = np.array(raw_buf)[::RAW_DECIMATE] line_raw.set_data(x_raw_dec[:len(arr)], arr) if thresh_long is not None: thresh_l_line.set_ydata([thresh_long, thresh_long]) thresh_l_line.set_alpha(0.5) score_color = NEON_RED if latest_score >= 65 else NEON_AMBER if latest_score >= SCORE_ALARM else NEON_GREEN live_score_txt.set_text(f"SCORE: {latest_score:3d}") live_score_txt.set_color(score_color) bend_label, bend_color = ("[!] BEND", NEON_RED) if latest_bend else ("[OK]", NEON_GREEN) live_bend_txt.set_text(f"FLAG: {bend_label}") live_bend_txt.set_color(bend_color) sc = list(score_buf) line_score.set_data(np.arange(len(sc)), sc) line_score.set_color(score_color) if latest_score > peak_score: peak_score = latest_score peak_score_txt.set_text(f"PEAK: {peak_score:3d}") peak_score_txt.set_color(NEON_RED if peak_score >= 65 else NEON_AMBER if peak_score >= SCORE_ALARM else DIM) # ── hold timer → fires alert ── now = time.time() if not calibrating and latest_score >= SCORE_ALARM: if _bad_hold_start is None: _bad_hold_start = now held = now - _bad_hold_start hold_txt.set_text(f"Bent {held:.1f}s / {ALERT_HOLD_SEC}s") hold_txt.set_color(NEON_RED if held > ALERT_HOLD_SEC * 0.6 else NEON_AMBER) if held >= ALERT_HOLD_SEC: fire_alert(latest_score) _bad_hold_start = now # avoid re-firing every frame; cooldown also gates this else: _bad_hold_start = None hold_txt.set_text("") alerts_txt.set_text(f"ALERTS: {session_alerts}") # ── session timer ── elapsed = int(now - session_start) h, rem = divmod(elapsed, 3600) m, s = divmod(rem, 60) session_timer_txt.set_text(f"T+{h:02d}:{m:02d}:{s:02d}") # ── CRT scanline sweep — cheap continuous drift, wraps top→bottom ── _scan_y[0] -= 0.014 if _scan_y[0] < -0.02: _scan_y[0] = 1.0 scan_rect.set_y(_scan_y[0]) # ── alarm flash overlay ── (banner disabled; keep counters/decay logic intact) if alert_flash > 0: alert_flash -= 1 alarm_scrim.patch.set_alpha(0) for t in _ALARM_ARTISTS: t.set_alpha(0) return _ANIMATED_ARTISTS def update(frame): global _frame_error_logged try: return _update_impl(frame) except Exception: if not _frame_error_logged: print("\n[UPDATE ERROR] Frame error — recovering.\n") traceback.print_exc() _frame_error_logged = True return _ANIMATED_ARTISTS # ══════════════════════════════════════════════════════════════════════ # MAIN # ══════════════════════════════════════════════════════════════════════ if __name__ == "__main__": if PORT is None: detected = find_arduino_port() if detected: PORT = detected print(f"[AUTO-DETECT] Found Arduino on {PORT}") else: print("\n[ERROR] No Arduino port found automatically.") print(" → Plug in the Arduino, then re-run, or set PORT = 'COMx' at the top.\n") sys.exit(1) print(f""" ╔══════════════════════════════════════════════════════╗ ║ NECK-BEND POSTURE MONITOR v8.0 · starting… ║ ╠══════════════════════════════════════════════════════╣ ║ Port : {PORT:<40}║ ║ Baud : {BAUD:<40}║ ║ Threshold : score ≥ {SCORE_ALARM} held {ALERT_HOLD_SEC}s{'':<22}║ ║ Cooldown : {ALERT_COOLDOWN}s between alerts{'':<25}║ ║ Window : {DISPLAY_SECS}s EMG / {SCORE_HISTORY}pt score history{'':<12}║ ║ Refresh : {1000/FPS_MS:.0f}fps ({FPS_MS}ms/frame){'':<24}║ ╚══════════════════════════════════════════════════════╝ """) try: ser = open_port(PORT, BAUD) print(f"[OK] Connected to Arduino on {PORT}") except serial.SerialException as e: print(f"\n[ERROR] Cannot open {PORT} after {PORT_OPEN_RETRIES} attempts:\n {e}") print(" → This usually means another program already has the port open:") print(" - Close Arduino IDE's Serial Monitor / Serial Plotter") print(" - Make sure no earlier copy of this script is still running") print(" - Unplug/replug the Arduino, or try a different USB port") sys.exit(1) threading.Thread(target=serial_reader, daemon=True).start() # blit=False: several HUD artists (fig.text() labels, the alarm overlay patch) # aren't attached to any single Axes, so matplotlib's blit path crashes trying # to look up ax._get_view() for them. Full redraw avoids that entirely. ani = animation.FuncAnimation(fig, update, interval=FPS_MS, blit=False, cache_frame_data=False) print(""" HOW TO USE: 1. Upload posture_monitor.ino to the Arduino first. 2. Electrodes on UPPER TRAPEZIUS: IN+/IN- 2cm apart on the muscle, REF on a bony point (elbow/wrist). 3. Sit UPRIGHT, relaxed, for 4s while it calibrates (LED blinks). 4. LED solid = monitoring. Bend your neck forward → alarm after it's held past the threshold. SENSITIVITY (this file, top of CONFIG section): SCORE_ALARM lower = more sensitive (now 20, was 28) ALERT_HOLD_SEC shorter = fires faster (now 0.6s, was 1.0s) ALERT_COOLDOWN longer = fewer repeats (now 6s, was 8s) """) try: plt.show() except KeyboardInterrupt: pass finally: if ser and ser.is_open: ser.close() for p in _WAV_FILES.values(): try: os.unlink(p) except Exception: pass print("\n── Session ended ──") print(f" Total alerts : {session_alerts}")

## 5.3  Save and run the script

- Open Notepad (Windows) or TextEdit (Mac) or any text editor.

- Copy the entire script above into it exactly.

- Save the file as:  posture_monitor.py  inside your PostureMonitor folder.

- Open your terminal, navigate to the folder: type  cd Desktop\PostureMonitor  and press Enter.

- Run the script: type  python posture_monitor.py  and press Enter.

- A graph window should open and you'll see the signal coming in!

Common error: ‘Access is denied’ or the port won't open This means something else already has the COM port open, usually Arduino IDE's Serial Monitor/Plotter, or a previous run of this script that didn't exit cleanly. The script now retries automatically for about 7 to 8 seconds with a message telling you what to close. If it still fails: close the Arduino IDE's Serial Monitor, make sure no other copy of posture_monitor.py is running, then try again.

# Section 6: Calibration & Fine-Tuning Your Sensitivity

The good news: calibration now happens automatically. Every time you power up the Arduino, it spends 4 seconds measuring YOUR resting muscle signal and uses that as the baseline for everything else. No manual number-crunching needed. What you might still want to tune is how sensitive the alert is, which takes a few seconds in the Python script.

Why calibrate automatically? Every person's muscles are different. Your electrode placement is different. Even the thickness of your skin affects the signal. There's no single magic number that works for everyone, so the Arduino measures YOUR resting baseline fresh every time you power it on. Every threshold in the code is then calculated relative to that baseline (for example, “1.4× baseline”) rather than a fixed number.

Step 1: Let the Arduino auto-calibrate Power up the Arduino (or press its reset button) with the electrodes already attached. Sit up straight and keep your neck and shoulder muscles fully relaxed for the full 4 seconds. The onboard LED blinks fast while this happens. When the LED goes solid, calibration is done and monitoring has started.

Step 2: Watch your live posture score Run posture_monitor.py and watch the SCORE readout on the raw EMG panel. Sit well and the score should stay low and green, well under SCORE_ALARM (20 by default). Now hunch forward deliberately. The score should climb into amber/red, and after it's held past ALERT_HOLD_SEC (0.6s by default) the alert should fire.

Step 3: Adjust sensitivity if needed All the sensitivity settings live in the CONFIG section near the top of posture_monitor.py. SCORE_ALARM: lower means more sensitive (fires on smaller bends). Default is 20. ALERT_HOLD_SEC: how long the bend must be sustained before it counts. Default is 0.6s. ALERT_COOLDOWN: minimum seconds between repeated alerts. Default is 6s. Change a value, save the file, and re-run the script to test.

Example: if the alarm fires during small movements like swallowing or talking, try raising SCORE_ALARM from 20 to around 28, or increasing ALERT_HOLD_SEC so brief tension has to be sustained longer before it counts as a real bend.

Tip: Start sensitive, then relax it It's better to start with the default SCORE_ALARM (which errs toward catching real sustained bends) and only raise it if you're getting false alarms from talking, swallowing, or shifting in your seat. You'll get annoyed at first but your posture will improve fast!

# Section 7: Troubleshooting Common Problems

Don't panic when something doesn't work the first time. That's completely normal! Here's a guide to the most common problems:

| Problem | Likely cause | Fix |
| --- | --- | --- |
| Signal is flat (all same number) | Electrodes not connected or not touching skin | Re-press each electrode. Check cable snaps. Try a new electrode. |
| Signal is very noisy / chaotic | Reference electrode not on a bony area | Move REF to the back of your elbow bone or wrist bone |
| ‘Access is denied’ opening the port | Another program already has the COM port open | Close Arduino IDE's Serial Monitor/Plotter and any other running copy of this script. The script retries automatically for about 7 to 8 seconds first |
| 'Module not found' Python error | Library not installed properly | Run the pip install command again in the terminal |
| Arduino LED not on | USB not connected / wrong cable | Try a different USB port on your laptop. Ensure cable supports data (not charge-only) |
| Alerts fire on small movements (talking, swallowing) | SCORE_ALARM or ALERT_HOLD_SEC too sensitive | Raise SCORE_ALARM (try 28) or increase ALERT_HOLD_SEC in the CONFIG section, then re-test |
| Alerts never fire even when slouching | Threshold too high, electrodes slipped, or you calibrated while already tense | Lower SCORE_ALARM, check electrode adhesion, and re-power the Arduino to recalibrate while sitting fully relaxed |
| Graph window won't open | matplotlib not installed | Run: pip install matplotlib --upgrade |

# Section 8: Quick Reference Card

Print this page and keep it next to your workspace!

## Hardware connections at a glance

| Connection | Where |
| --- | --- |
| IN+ (Red snap) | Right trapezius muscle belly (back of neck/shoulder) |
| IN- (Black snap) | Same muscle, 2 cm to the right of IN+ |
| REF (Green snap) | Bony area (back of elbow bone) |
| BioAmp Cable plug | 3-pin header on Muscle BioAmp Shield (labelled IN+, IN-, REF) |
| Shield | Stacked directly on top of Arduino Uno R4 |
| USB-C cable | Arduino Uno R4 to laptop USB port |

## Settings in the Python script

| Setting | What to change |
| --- | --- |
| PORT | Your COM port, e.g. COM3 (Windows) or /dev/ttyACM0 (Linux/Mac) |
| SCORE_ALARM | 20. Lower means more sensitive. Raise it if small movements trigger alerts |
| ALERT_HOLD_SEC | 0.6. How many seconds the score must stay over SCORE_ALARM before it counts as a real bend |
| ALERT_COOLDOWN | 6. Seconds between repeated alerts, so it doesn't spam you |
| DISPLAY_SECS | 25. How many seconds of EMG history the top graph shows before scrolling |

## Terminal commands

| Command | What it does |
| --- | --- |
| pip install pyserial numpy matplotlib sounddevice | Install all Python libraries (run once) |
| python posture_monitor.py | Start the posture monitor |
| Ctrl+C | Stop the program in the terminal |
| cd Desktop\PostureMonitor | Navigate to your project folder (Windows) |
| cd ~/Desktop/PostureMonitor | Navigate to your project folder (Mac/Linux) |

# Section 9: What to Try Next

Once your basic posture monitor is working, here are some cool things you can add or explore:

## Upgrades to the software

- Save data to a CSV file: log every session and open it in Excel to see your posture trends over weeks

- Build a web dashboard: use Flask (a Python web library) to show your live signal in a browser

## Upgrades to the hardware

- 3D print a headband holder so you don't need to stick electrodes to your neck every time

- Add a small buzzer (piezo buzzer) to the Arduino so you get a physical buzz instead of a screen notification (useful when headphones are on)

- Make it wireless: add an HC-05 Bluetooth module to the Arduino so you can walk around untethered

## Learn more about the science

- Protocentral BioAmp documentation: search 'Protocentral Upside Down Kit GitHub' for full datasheets and example sketches

- Backyard Brains (backyardbrains.com) has brilliant free neuroscience experiments for all ages

- Khan Academy has free videos on how muscles work electrically if you want to go deeper

You just built a real biomedical device! Seriously, the technology you just built is the same technology used in professional physical therapy clinics, sports science labs, and rehabilitation centres. EMG biofeedback (using muscle signals to give real-time feedback) is a well-established medical technique. You now understand how it works from the electrode on your skin all the way through to the notification on your screen. That's a long journey through biology, electronics, and software, and you did it all yourself. Nice work!

End of Guide